ASSIGNMENT REPORT

Digital Design

Asynchronous FIFO Depth Calculation

| Course | Digital Design |
| --- | --- |
| Assignment | FIFO Depth Calculation – Examples 1, 2 & 3 |
| Student Name | Mohamed Hossam El-Sawy |
| Instructor / TA | Eng. Ali Eltemsah |
| Submission Date | August 30, 2026 |

# 01   OBJECTIVE

This assignment determines the minimum required depth of an asynchronous FIFO that transfers a fixed-size burst of data between two modules running on independent clock domains, for three scenarios that differ in the idle (wait) cycles present on the write and read sides.

# 02   PROBLEM STATEMENT / REQUIREMENTS

Three worked examples are analyzed for an asynchronous FIFO transferring data from a faster writing module (A) to a slower reading module (B). In each case the FIFO must be sized so that it never overflows during the burst. The common and per-example parameters are:

- Writing clock f_A = 100 MHz (T_A = 10 ns); Reading clock f_B = 40 MHz (T_B = 25 ns); Burst Length (BL) = 100 data items — common to all three examples.

- Example 1: no idle cycles on either the write side or the read side (back-to-back writes and reads).

- Example 2: 2 idle cycles between successive writes and 2 idle cycles between successive reads.

- Example 3: 2 idle cycles between successive writes and 1 idle cycle between successive reads.

# 03   DESIGN & ANALYSIS

## 3.1 Design Approach

Because f_A > f_B in every example, module A always writes faster than module B can read, so FIFO occupancy grows throughout the burst and peaks when the last item is written. The minimum depth is found in three steps: (1) find the effective write period T_A' and read period T_B' by adding any idle cycles to the base period; (2) find the total time needed to write the whole burst, T_W = BL × T_A'; (3) find how many items B has managed to read during that same time, N_read = T_W / T_B'. The minimum required FIFO depth is then Depth = BL − N_read.

## 3.2 Given Data & Calculation

The table below lists the parameters and intermediate results for each example, derived using the method described in 3.1.

| Parameter | Example 1 | Example 2 | Example 3 |
| --- | --- | --- | --- |
| Write clock, f_A | 100 MHz | 100 MHz | 100 MHz |
| Read clock, f_B | 40 MHz | 40 MHz | 40 MHz |
| Write period, T_A | 10 ns | 10 ns | 10 ns |
| Read period, T_B | 25 ns | 25 ns | 25 ns |
| Idle cycles / write | 0 | 2 | 2 |
| Idle cycles / read | 0 | 2 | 1 |
| Effective write period, T_A' | 10 ns | 30 ns | 30 ns |
| Effective read period, T_B' | 25 ns | 75 ns | 50 ns |
| Burst Length, BL | 100 | 100 | 100 |
| Total write time, T_W = BL × T_A' | 1000 ns | 3000 ns | 3000 ns |
| Items read, N_read = T_W / T_B' | 40 | 40 | 60 |
| Minimum FIFO depth = BL − N_read | 60 | 60 | 40 |

Table 1: Calculation parameters and results for Examples 1, 2 and 3.

## 3.3 Async FIFO Architecture

All three examples share the same generic architecture: module A writes into the FIFO on its own clock domain, and module B reads from the FIFO on an independent, slower clock domain, as shown below.

Figure 1: Generic asynchronous FIFO connecting Module A (write clock domain) and Module B (read clock domain).

## 3.4 Step-by-Step Calculation

The arithmetic performed for each example, following the method in 3.1, is shown below.

Example 1 (0 idle cycles write, 0 idle cycles read):

T_W = BL x T_A = 100 x 10 ns = 1000 ns

N_read = T_W / T_B = 1000 / 25 = 40 items

Depth = BL - N_read = 100 - 40 = 60 items

Example 2 (2 idle cycles write, 2 idle cycles read):

T_A' = 3 x T_A = 30 ns      T_B' = 3 x T_B = 75 ns

T_W = BL x T_A' = 100 x 30 ns = 3000 ns

N_read = T_W / T_B' = 3000 / 75 = 40 items

Depth = BL - N_read = 100 - 40 = 60 items

Example 3 (2 idle cycles write, 1 idle cycle read):

T_A' = 3 x T_A = 30 ns      T_B' = 2 x T_B = 50 ns

T_W = BL x T_A' = 100 x 30 ns = 3000 ns

N_read = T_W / T_B' = 3000 / 50 = 60 items

Depth = BL - N_read = 100 - 60 = 40 items

# 04   RESULTS SUMMARY

The table below summarizes the minimum FIFO depth obtained for each example.

| Example | Idle Cycles (Write / Read) | Items Read During Burst | Minimum FIFO Depth |
| --- | --- | --- | --- |
| 1 | 0 / 0 | 40 | 60 |
| 2 | 2 / 2 | 40 | 60 |
| 3 | 2 / 1 | 60 | 40 |

# 05   DISCUSSION

Examples 1 and 2 give the same minimum depth (60 items) even though idle cycles are introduced in Example 2, because the ratio between the effective read and write periods is unchanged (T_B'/T_A' = 2.5 in both cases) — adding equal proportional idle time on both sides does not change the relative speed difference between A and B. Example 3 requires a smaller depth (40 items) because the read side has fewer idle cycles than the write side, so B reads relatively faster compared to A than in Examples 1 and 2, and fewer items accumulate in the FIFO before B catches up. In a real design, these minimum depths would typically be rounded up to the next convenient binary size (e.g. 64) since FIFO read/write pointers are usually implemented with binary or Gray-code counters.

# 06   CONCLUSION

Using a burst-time / read-time analysis, the minimum FIFO depth needed to avoid overflow was found to be 60 items for Examples 1 and 2, and 40 items for Example 3. The key takeaway is that the required FIFO depth depends on the relative effective throughput of the write and read clock domains — including any idle cycles on either side — rather than on the raw clock frequencies alone.

# A   APPENDIX – HANDWRITTEN CALCULATIONS

Scanned original hand-written calculations for Examples 1, 2 and 3, corresponding to the results presented in Sections 3.2–3.4 above.