ASSIGNMENT REPORT

Digital Design

Assignment 5.1 - Automatic Garage Door Controller (Moore FSM)

| Course | Digital Design |
| --- | --- |
| Assignment | Assignment 5.1 - Automatic Garage Door Controller |
| Student Name | Mohamed Hossam El-Sawy |
| Instructor / TA | Eng. Ali Temsah |
| Submission Date | July 26, 2026 |

| 01 | OBJECTIVE |
| --- | --- |

This assignment designs, implements and verifies an Automatic Garage Door Controller as a Moore finite state machine (FSM). The controller drives an Up motor and a Down motor from two limit-sensor inputs and a single user push-button, and is initialised to a safe IDLE state through an asynchronous reset.

| 02 | PROBLEM STATEMENT / REQUIREMENTS |
| --- | --- |

A Moore FSM must be designed and verified with the following behaviour:

- Three inputs: Activate (user push-button), UP_Max (sensor, high when the door is fully open), DN_Max (sensor, high when the door is fully closed).

- Two outputs: UP_M (drives the Up motor) and DN_M (drives the Down motor).

- The door is always either fully open or fully closed - Activate triggers the Up motor when the door is down, and the Down motor when the door is up.

- The FSM is initialised to the IDLE state using an asynchronous reset (RST).

- Three states: IDLE (default/reset state, both motors off), Mv_Up (Up motor enabled to open the door), Mv_Dn (Down motor enabled to close the door).

- Deliverables: the Verilog implementation of the Moore FSM, and a self-checking testbench that validates the design at a 50 MHz clock frequency.

| 03 | DESIGN & ANALYSIS |
| --- | --- |

3.1 Design Approach

The controller is coded as a classic two-always-block Moore FSM. current_state is a 2-bit register updated on posedge CLK, with an asynchronous active-low reset (negedge RST in the sensitivity list) that forces current_state back to IDLE_state regardless of the clock. A second, purely combinational always block computes next_state from current_state and the inputs using a case statement. Because this is a Moore machine, the outputs UP_M and DN_M are continuous assign statements that depend only on current_state (UP_M = current_state == Mv_Up_state, DN_M = current_state == Mv_Dn_state) - they never depend directly on the inputs, so they only change on a clock edge together with the state.

From IDLE, the design interprets which sensor is already asserted to decide direction: if the door is closed (DN_Max=1, UP_Max=0) and Activate is pressed, the FSM opens the door via Mv_Dn/Mv_Up encoding matched to the state diagram; if the door is open (UP_Max=1, DN_Max=0) and Activate is pressed, it moves the other way. If both sensors read the same value (a broken/ambiguous sensor condition) the FSM stays safely in IDLE. Once moving, Mv_Up and Mv_Dn ignore Activate and only return to IDLE when their respective limit sensor (UP_Max or DN_Max) is reached.

3.2 State Tables

Moore output table - UP_M/DN_M depend only on the present state:

| State | Encoding | UP_M | DN_M |
| --- | --- | --- | --- |
| IDLE | 00 | 0 | 0 |
| Mv_Up | 01 | 1 | 0 |
| Mv_Dn | 10 | 0 | 1 |

Table 1: Moore output table for Automatic_Garage_Door_Controller

Next-state table:

| Current State | Condition | Next State |
| --- | --- | --- |
| IDLE | Activate = 0 | IDLE |
| IDLE | Activate = 1, UP_Max = 1, DN_Max = 0 | Mv_Dn |
| IDLE | Activate = 1, UP_Max = 0, DN_Max = 1 | Mv_Up |
| IDLE | Activate = 1, UP_Max = DN_Max (both sensors agree/broken) | IDLE |
| Mv_Up | UP_Max = 0 | Mv_Up (hold) |
| Mv_Up | UP_Max = 1 | IDLE |
| Mv_Dn | DN_Max = 0 | Mv_Dn (hold) |
| Mv_Dn | DN_Max = 1 | IDLE |

Table 2: Next-state transition table

Top-level port list:

| Signal Name | Direction | Width (bits) | Description |
| --- | --- | --- | --- |
| UP_Max | input | 1 | Sensor - high when the door is completely open |
| DN_Max | input | 1 | Sensor - high when the door is completely closed |
| Activate | input | 1 | User push-button |
| CLK | input | 1 | System clock |
| RST | input | 1 | Asynchronous active-low reset |
| UP_M | output | 1 | Drives the Up motor |
| DN_M | output | 1 | Drives the Down motor |

Table 3: Automatic_Garage_Door_Controller port description

3.3 Block Diagram and State Diagram

Figure 1: Automatic Garage Door Controller block interface

Figure 2: Moore state diagram - IDLE, Mv_Up and Mv_Dn states

3.4 HDL Implementation

Automatic_Garage_Door_Controller.v - full source (state register, asynchronous reset, Moore outputs and next-state logic):

module Automatic_Garage_Door_Controller ( input UP_Max,DN_Max, input Activate, input CLK,RST, output UP_M,DN_M ); localparam [1:0] IDLE_state = 0, Mv_Up_state = 1, Mv_Dn_state = 2; reg [1:0] next_state,current_state; always @(posedge CLK or negedge RST) begin if (!RST) begin current_state <= IDLE_state; end else current_state <= next_state; end assign UP_M = current_state == Mv_Up_state; assign DN_M = current_state == Mv_Dn_state; always @(*) begin next_state = IDLE_state; case (current_state) IDLE_state: if (!Activate) begin next_state = IDLE_state; end else if (UP_Max && !DN_Max) begin next_state = Mv_Dn_state; end else if (!UP_Max && DN_Max) begin next_state = Mv_Up_state; end else next_state = IDLE_state; Mv_Up_state: if (UP_Max) begin next_state = IDLE_state; end else next_state = Mv_Up_state; Mv_Dn_state: if (DN_Max) begin next_state = IDLE_state; end else next_state = Mv_Dn_state; endcase end endmodule //Automatic_Garage_Door_Controller

| 04 | SIMULATION & RESULTS |
| --- | --- |

Verification approach: Automatic_Garage_Door_Controller_TB.v is a self-checking testbench that drives UP_Max, DN_Max and Activate on the negedge of a 50 MHz clock (CLK_PERIOD = 20 ns) and checks DUT.current_state, UP_M and DN_M on the following negedge against expected values, reporting PASS/FAIL through reusable apply_reset / drive_inputs / check_state_outputs tasks. The suite covers 19 test cases: asynchronous reset, IDLE hold, both direction transitions (open/close), holding state while a sensor is still low, returning to IDLE once a limit sensor is reached, IDLE holding when both sensors read the same (ambiguous) value, Mv_Up/Mv_Dn correctly ignoring Activate while moving, and asynchronous reset asserted mid-cycle from both Mv_Up and Mv_Dn.

4.1 Test Case Summary

| # | Test Case | Time (ns) | Result |
| --- | --- | --- | --- |
| 1 | Async Reset -> IDLE | 60000 | PASSED |
| 2 | IDLE hold (Activate=0) | 100000 | PASSED |
| 3 | IDLE -> Mv_Up (open door) | 140000 | PASSED |
| 4 | Mv_Up hold (UP_Max=0) | 180000 | PASSED |
| 5 | Mv_Up -> IDLE (UP_Max=1) | 220000 | PASSED |
| 6 | IDLE -> Mv_Dn (close door) | 260000 | PASSED |
| 7 | Mv_Dn hold (DN_Max=0) | 300000 | PASSED |
| 8 | Mv_Dn -> IDLE (DN_Max=1) | 340000 | PASSED |
| 9 | IDLE hold (both sensors low) | 380000 | PASSED |
| 10 | IDLE hold (both sensors high) | 420000 | PASSED |
| 11 | Enter Mv_Up for ignore test | 460000 | PASSED |
| 12 | Mv_Up ignores Activate | 500000 | PASSED |
| 13 | Return to IDLE | 540000 | PASSED |
| 14 | Enter Mv_Dn for ignore test | 580000 | PASSED |
| 15 | Mv_Dn ignores Activate | 620000 | PASSED |
| 16 | Async Reset from Mv_Dn | 666000 | PASSED |
| 17 | IDLE cleanup after async reset | 700000 | PASSED |
| 18 | Enter Mv_Up for reset test | 740000 | PASSED |
| 19 | Async Reset from Mv_Up | 766000 | PASSED |

Table 4: Summary of all 19 directed test cases (extracted from the simulation log) - 19/19 PASSED

4.2 Simulation Waveform

ModelSim waveform showing current_state/next_state, UP_Max, DN_Max, Activate, CLK, RST and the UP_M/DN_M outputs cycling through IDLE -> Mv_Up -> IDLE -> Mv_Dn -> IDLE as Activate and the limit sensors are exercised:

Figure 3: ModelSim simulation waveform for Automatic_Garage_Door_Controller_TB

4.3 Full Simulation Log

Console transcript produced by Automatic_Garage_Door_Controller_TB.v, showing every PASS result and the final tally (19/19 passed, 0 failed):

================================================================================ Automatic Garage Door Controller - Self-Checking Testbench ================================================================================ [INFO] Clock Period = 20 ns (50 MHz) [INFO] Applying asynchronous reset... TEST 1 PASSED: Async Reset -> IDLE @ 60000 TEST 2 PASSED: IDLE hold (Activate=0) @ 100000 TEST 3 PASSED: IDLE -> Mv_Up (open door) @ 140000 TEST 4 PASSED: Mv_Up hold (UP_Max=0) @ 180000 TEST 5 PASSED: Mv_Up -> IDLE (UP_Max=1) @ 220000 TEST 6 PASSED: IDLE -> Mv_Dn (close door) @ 260000 TEST 7 PASSED: Mv_Dn hold (DN_Max=0) @ 300000 TEST 8 PASSED: Mv_Dn -> IDLE (DN_Max=1) @ 340000 TEST 9 PASSED: IDLE hold (both sensors low) @ 380000 TEST 10 PASSED: IDLE hold (both sensors high) @ 420000 TEST 11 PASSED: Enter Mv_Up for ignore test @ 460000 TEST 12 PASSED: Mv_Up ignores Activate @ 500000 TEST 13 PASSED: Return to IDLE @ 540000 TEST 14 PASSED: Enter Mv_Dn for ignore test @ 580000 TEST 15 PASSED: Mv_Dn ignores Activate @ 620000 TEST 16 PASSED: Async Reset from Mv_Dn @ 666000 TEST 17 PASSED: IDLE cleanup after async reset @ 700000 TEST 18 PASSED: Enter Mv_Up for reset test @ 740000 TEST 19 PASSED: Async Reset from Mv_Up @ 766000 ================================================================================ TEST SUMMARY ================================================================================ Total Tests Run : 19 Passed : 19 Failed : 0 -------------------------------------------------------------------------------- RESULT : ALL TESTS PASSED ================================================================================ ** Note: $finish : Automatic_Garage_Door_Controller_TB.v(287) Time: 766 ns Iteration: 0 Instance: /Automatic_Garage_Door_Controller_TB

Log 1: Full ModelSim transcript for Automatic_Garage_Door_Controller_TB

| 05 | DISCUSSION |
| --- | --- |

All 19 directed test cases passed. The asynchronous reset behaves correctly both at power-on and when asserted mid-cycle from Mv_Up and Mv_Dn (Tests 16 and 19), confirming current_state returns to IDLE_state independently of CLK as required. The Moore-output property was specifically exercised by holding Mv_Up/Mv_Dn with Activate still asserted (Tests 12 and 15): UP_M and DN_M remained correct and unaffected by the stale Activate input, since they are combinationally tied only to current_state. The ambiguous-sensor cases (Tests 9 and 10, both sensors low or both high) confirm the FSM fails safe by remaining in IDLE rather than committing to a direction it cannot verify. No mismatches or timing violations were observed over the 766 ns simulation.

| 06 | CONCLUSION |
| --- | --- |

A Moore FSM was designed and verified for the Automatic Garage Door Controller, with three states (IDLE, Mv_Up, Mv_Dn), asynchronous active-low reset, and outputs registered purely as a function of state. A 19-case self-checking testbench running at 50 MHz confirmed correct state transitions in both directions, correct holding behaviour while a door is mid-travel, correct handling of ambiguous sensor readings, correct ignoring of the Activate input while moving, and correct asynchronous reset recovery from any state.