ASSIGNMENT REPORT

Digital Design

Assignment 2.0 / 2.1 / 2.2 - Tcl Scripting for EDA Automation

| Course | Digital Design |
| --- | --- |
| Assignment | Assignment 2 - Tcl Scripting (2.0, 2.1, 2.2) |
| Student Name | Mohamed Hossam El-Sawy |
| Instructor / TA | Eng. Ali Temsah |
| Submission Date | July 24, 2026 |

| 01 | OBJECTIVE |
| --- | --- |

This assignment introduces Tcl scripting as it is used to automate repetitive tasks around EDA tool flows. Three short scripts are developed: Ass2_0.tcl practises string/list manipulation (title-casing a list of city names), Ass2_1.tcl generates a Verilog module port-list declaration automatically from a set of Tcl variables/lists describing port names and widths, and Ass2_2.tcl reads a list of RTL file names from a text file and reformats it into a single Tcl list that EDA tool commands (e.g. read_verilog / analyze) can consume directly.

| 02 | PROBLEM STATEMENT / REQUIREMENTS |
| --- | --- |

Assignment 2.0 - List Manipulation

- Create a Tcl script named Ass2_0.tcl.

- Print the message "****Assignment 2.0****".

- Create a list named cities: cairo, alexandria, damietta, dakahlia, faiyum, sohag, aswan.

- Using a foreach loop, build a new list Cities_New where each element is title-cased (first letter upper, rest lower).

Assignment 2.1 - Verilog Interface Generator

- Create a Tcl script named Ass2_1.tcl.

- Print the message "****Writing Verilog Block Interface****".

- Create variable modname = "Up_Dn_Counter".

- Create in_ports = {IN, Load, Up, Down, CLK} and in_ports_width = {4, 1, 1, 1, 1} (matching position).

- Create out_ports = {High, Counter, Low} and out_ports_width = {1, 4, 1} (matching position).

- Generate and print the Verilog module port-list declaration, choosing a plain "input"/"output" for width-1 signals and a "[width-1:0]" range otherwise, with a trailing comma on every port except the last.

Assignment 2.2 - RTL File List Parser

- Create a Tcl script named Ass2_2.tcl.

- Read the RTL file names listed in rtl.txt (Async_fifo.v, DF_Sync.v, fifo_mem.v, fifo_rd.v, fifo_wr.v, Int_ClkDiv.v, enable_sync.v).

- Store the file names as the elements of a list named designs.

- Print designs in curly-brace Tcl list form: {Async_fifo.v DF_Sync.v fifo_mem.v fifo_rd.v fifo_wr.v Int_ClkDiv.v enable_sync.v}.

| 03 | DESIGN & ANALYSIS |
| --- | --- |

3.1 Design Approach

Ass2_0.tcl relies on Tcl's built-in "string totitle" command, which already returns a string with only its first character upper-cased and the remainder lower-cased, so a single call per element inside a foreach loop is enough; each result is appended to Cities_New with lappend.

Ass2_1.tcl treats the four port lists as parallel arrays: the index of a name in in_ports/out_ports is the same index used to look up its width in in_ports_width/out_ports_width via lindex. A running port_index counter (against the pre-computed total_ports = llength(in_ports)+llength(out_ports)) decides whether a trailing comma is needed. A width of 1 prints a bare input/output; any other width prints a [width-1:0] range using expr.

Ass2_2.tcl opens rtl.txt with open/read, which returns the whole file as one Tcl string already split into whitespace-separated "words" - conveniently this is exactly Tcl's native list format, so the string can be treated directly as the designs list. It is then reprinted with "[join $designs " "]" wrapped in braces to match the required {...} output format.

3.2 Data Structures

Assignment 2.0 - cities -> Cities_New mapping (string totitle):

| cities (input) | Cities_New (output) |
| --- | --- |
| cairo | Cairo |
| alexandria | Alexandria |
| damietta | Damietta |
| dakahlia | Dakahlia |
| faiyum | Faiyum |
| sohag | Sohag |
| aswan | Aswan |

Table 1: Title-casing performed by Ass2_0.tcl

Assignment 2.1 - port name / direction / width used to generate the module header:

| Port | Direction | Width (bits) |
| --- | --- | --- |
| IN | input | 4 |
| Load | input | 1 |
| Up | input | 1 |
| Down | input | 1 |
| CLK | input | 1 |
| High | output | 1 |
| Counter | output | 4 |
| Low | output | 1 |

Table 2: Port list consumed by Ass2_1.tcl to generate Up_Dn_Counter's interface

3.3 File / Data Flow

rtl.txt (plain list of seven .v file names, one per line) is read as a single string by Ass2_2.tcl and re-emitted as the Tcl list designs, ready to be looped over by later EDA-tool automation (e.g. foreach f $designs { read_verilog $f }):

Async_fifo.v DF_Sync.v fifo_mem.v fifo_rd.v fifo_wr.v Int_ClkDiv.v enable_sync.v

Figure 1: Contents of rtl.txt consumed by Ass2_2.tcl

3.4 Tcl Implementation

Ass2_0.tcl:

puts "****Assignment 2.0****"; set cities [list cairo alexandria damietta dakahlia faiyum sohag aswan]; set Cities_New {}; foreach City $cities { lappend Cities_New [string totitle $City]; } puts "$Cities_New";

Ass2_1.tcl:

puts "****Writing Verilog Block Interface****"; set modname "Up_Dn_Counter"; set in_ports [list IN Load Up Down CLK]; set in_ports_width [list 4 1 1 1 1 ]; set out_ports [list High Counter Low]; set out_ports_width [list 1 4 1 ]; puts "module $modname ("; set inputs_size [llength $in_ports]; set output_size [llength $out_ports]; set total_ports [expr {$inputs_size + $output_size}]; set port_index 0; # Print inputs for {set index 0} {$index < $inputs_size} {incr index} { set name [lindex $in_ports $index]; set width [lindex $in_ports_width $index]; incr port_index; set comma [expr {$port_index < $total_ports ? "," : ""}]; if {$width == 1} { puts " input $name$comma"; } else { puts " input \[[expr {$width-1}]:0\] $name$comma"; } } # Print outputs for {set index 0} {$index < $output_size} {incr index} { set name [lindex $out_ports $index]; set width [lindex $out_ports_width $index]; incr port_index; set comma [expr {$port_index < $total_ports ? "," : ""}]; if {$width == 1} { puts " output $name$comma"; } else { puts " output \[[expr {$width-1}]:0\] $name$comma"; } } puts " );"

Ass2_2.tcl:

set fh [open rtl.txt r]; set designs [read $fh]; close $fh; puts "{[join $designs " "]}";

| 04 | SIMULATION & RESULTS |
| --- | --- |

Each script was executed with tclsh from the command line. The console output below was captured directly from each run and matches the expected output given in the assignment statement in every case.

4.1 Ass2_0.tcl - Console Output

# ****Assignment 2.0**** # Cairo Alexandria Damietta Dakahlia Faiyum Sohag Aswan

Log 1: tclsh Ass2_0.tcl output

4.2 Ass2_1.tcl - Console Output

# ****Writing Verilog Block Interface**** # module Up_Dn_Counter ( # input [3:0] IN, # input Load, # input Up, # input Down, # input CLK, # output High, # output [3:0] Counter, # output Low # );

Log 2: tclsh Ass2_1.tcl output - matches the expected Up_Dn_Counter interface

4.3 Ass2_2.tcl - Console Output

# {Async_fifo.v DF_Sync.v fifo_mem.v fifo_rd.v fifo_wr.v Int_ClkDiv.v enable_sync.v}

Log 3: tclsh Ass2_2.tcl output - matches the required {...} list format

4.4 Result Summary

| Script | Requirement | Result |
| --- | --- | --- |
| Ass2_0.tcl | Title-case all 7 city names via foreach | PASSED |
| Ass2_1.tcl | Auto-generate Up_Dn_Counter port list | PASSED |
| Ass2_2.tcl | Parse rtl.txt into designs list | PASSED |

Table 3: Result summary for all three scripts - 3/3 matched the expected output

| 05 | DISCUSSION |
| --- | --- |

All three scripts reproduced the expected output exactly. Ass2_0.tcl confirms that Tcl's string totitle removes the need for manual character-case handling. Ass2_1.tcl shows how parallel lists (name/width) plus lindex and a running index/comma counter are enough to auto-generate boilerplate HDL, which is directly useful for scripting port-list generation from a spreadsheet or CSV of signals. Ass2_2.tcl highlights that Tcl's "everything is a string" model means a whitespace-separated file read with a plain "read" is already usable as a Tcl list, so no manual splitting/parsing was required - only a join was needed to print it back out in the requested brace-delimited form. No discrepancies were observed between the produced and expected outputs for any of the three assignments.

| 06 | CONCLUSION |
| --- | --- |

Three small Tcl utilities were implemented and verified: a list/string transformation script (2.0), a parametric Verilog port-list generator driven entirely by Tcl variables and lists (2.1), and an RTL file-list loader that turns a flat text file into a ready-to-use Tcl list (2.2). Together they demonstrate the core Tcl constructs - foreach, string, list, lindex/llength, open/read, join and expr - that underpin most EDA tool automation scripts.