ASSIGNMENT REPORT

Digital Design

Assignment 5.0 — Linear Feedback Shift Register (LFSR) / CRC Generator

| Course | Digital Design |
| --- | --- |
| Assignment | Assignment 5.0 — Linear Feedback Shift Register (LFSR) based CRC Generator |
| Student Name | Mohamed Hossam El-Sawy |
| Instructor / TA | Eng. Ali Temsah |
| Submission Date | July 27, 2026 |

# 01  OBJECTIVE

This assignment aims to design, implement, and verify a Linear Feedback Shift Register (LFSR) used as a serial CRC generator. The design shifts in one byte of data (LSB first), computes an 8-bit CRC remainder in an 8-bit shift register, and then serially shifts the CRC bits out through a single-bit output port, with a Valid flag marking the CRC transmission window.

# 02  PROBLEM STATEMENT / REQUIREMENTS

The task is to build an 8-bit Galois-style LFSR/CRC block with the following interface and behaviour, and to verify it with a self-checking testbench:

- Inputs: DATA (serial bit), ACTIVE (data-transmission enable), CLK, RST (asynchronous, active-low).

- Outputs: CRC (serial output bit, registered) and Valid (high only while CRC bits are being shifted out).

- All 8 registers (R7–R0) are asynchronously reset to the seed value 8'hD8.

- DATA bits are shifted in LSB-first while ACTIVE is high; once the byte has been fully shifted in, R7–R0 hold the 8-bit CRC remainder.

- When ACTIVE goes low, the CRC bits are shifted out serially through CRC (R0 first, i.e. LSB first) while Valid is asserted.

- Clock frequency: 10 MHz (100 ns period).

- Testbench reads 10 test vectors from DATA_h.txt and the corresponding expected CRC values from Expec_Out_h.txt, drives the DUT, and self-checks every result.

# 03  DESIGN & ANALYSIS

## 3.1 Design Approach

The CRC block is built as an 8-bit Galois LFSR. Two always blocks operate on the same clock/reset: one implements the shift-register datapath (LFSR contents, CRC output, Valid flag) and the other implements a 4-bit Counter used only during the CRC-output phase to know when all 8 CRC bits have been shifted out (Counter_Max = (Counter == 8)).

While ACTIVE is high, every clock edge shifts DATA_XOR_R0_W1 = DATA ⊕ LFSR[0] into R7, and into every register whose position is marked with a '1' in the tap mask XOR_Bits = 8'b0100_0100 (taps after R6 and after R2), while the remaining registers simply shift toward R0 — this realises the same structure as the Fibonacci/Galois LFSR shown in the assignment's block diagram. Once ACTIVE drops, the design shifts LFSR[0] out onto CRC once per clock (R0 first, R7 last) for 8 cycles, raising Valid for that window and feeding 0 into R7 from then on.

## 3.2 Register / Tap Configuration

Instead of a truth table, the relevant design constants are summarised below, since the block behaves as a shift register rather than combinational logic:

| Parameter | Value |
| --- | --- |
| No_Registers | 8 |
| SEED (reset value) | 8'hD8 (1101_1000) |
| XOR_Bits (feedback taps) | 8'b0100_0100 → taps after R6 and R2 |
| Shift-in order | DATA, LSB first, into R7 side |
| Shift-out order | CRC bits, R0 first (LSB) → R7 last (MSB) |

## 3.3 Circuit / Block Diagram

Block interface of the CRC module, as given in the assignment:

Figure 1: CRC block interface — DATA, ACTIVE, CLK, RST in; CRC, Valid out.

Internal LFSR structure, showing the two feedback XOR gates tapped after R7 and R3 (matching XOR_Bits = 0100_0100):

Figure 2: 8-bit Galois LFSR block diagram used for the CRC computation.

## 3.4 HDL Implementation

The design is implemented in Verilog as module CRC (file CRC.v). The core feedback/shift datapath is shown below:

// Port Declaration of CRC Module

module CRC (

input DATA, ACTIVE,

input CLK, RST,

// Serial CRC Output

output reg CRC,

// Check The Output is Valid to See or No

output reg Valid

);

localparam [3:0] No_Registers = 8; // Set The Number of the registers

localparam [No_Registers - 1:0] SEED = 'hD8; // RST Default Value for the Register

localparam [No_Registers - 1:0] XOR_Bits = 'b0100_0100; // Set the XOR Gates Places

reg [No_Registers - 1:0] LFSR;  // Registers for the algorithm and shift register

reg [3:0] Counter;              // Counts output bits shifted so far

integer i;

wire DATA_XOR_R0_W1;  // Feedback signal

wire Counter_Max;     // Finished shifting out all CRC bits

assign Counter_Max = (Counter == No_Registers);

assign DATA_XOR_R0_W1 = DATA ^ LFSR[0];

// Outputs and registers setting

always @(posedge CLK or negedge RST) begin

if (!RST) begin

LFSR  <= SEED;

Valid <= 0;

end

else if (ACTIVE) begin

Valid <= 0;

LFSR[No_Registers - 1] <= DATA_XOR_R0_W1;

for (i = 0; i < No_Registers - 1; i = i + 1) begin

if (XOR_Bits[i])

LFSR[i] <= LFSR[i + 1] ^ DATA_XOR_R0_W1;

else

LFSR[i] <= LFSR[i + 1];

end

end

else if (!Counter_Max) begin

Valid <= 1;

CRC   <= LFSR[0];

LFSR[No_Registers - 1] <= 0;

for (i = 0; i < No_Registers - 1; i = i + 1) begin

LFSR[i] <= LFSR[i + 1];

end

end

else begin

Valid <= 0;

CRC   <= 0;

end

end

// Setting the Counter

always @(posedge CLK or negedge RST) begin

if (!RST) begin

Counter <= No_Registers;

end

else if (ACTIVE) begin

Counter <= 0;

end

else if (!Counter_Max) begin

Counter <= Counter + 1;

end

end

endmodule //CRC

# 04  SIMULATION & RESULTS

The design was verified in ModelSim using a self-checking testbench (CRC_TB.v) driven by a 10 MHz clock (100 ns period). The testbench reads 10 DATA bytes and their expected CRC values from DATA_h.txt / Expec_Out_h.txt, resets the DUT, drives each byte in LSB-first with ACTIVE high, waits for Valid to assert, samples 8 CRC bits (R0 first), and compares the result against the expected value, tallying passes and failures.

Key testbench tasks:

`timescale 1ns/1ps

module CRC_TB;

//-------------------------------------------------------------------------

// Parameters

//-------------------------------------------------------------------------

localparam CLK_PERIOD   = 100; // 10 MHz -> 100 ns period

localparam No_Registers = 8;   // No of registers in the algorithm

localparam No_Tests     = 10;  // no of test cases

reg [No_Registers - 1:0] DATA_input       [0:No_Tests - 1];

reg [No_Registers - 1:0] Expected_Out_CRC [0:No_Tests - 1];

//-------------------------------------------------------------------------

// Signals

//-------------------------------------------------------------------------

reg  DATA_TB, ACTIVE_TB, CLK_TB, RST_TB;

wire CRC_TB, Valid_TB;

integer test_num = 0;

integer passed   = 0;

integer failed   = 0;

//-------------------------------------------------------------------------

// DUT Instantiation

//-------------------------------------------------------------------------

CRC DUT_CRC (

.DATA(DATA_TB), .ACTIVE(ACTIVE_TB),

.CLK(CLK_TB), .RST(RST_TB),

.CRC(CRC_TB), .Valid(Valid_TB)

);

//-------------------------------------------------------------------------

// Clock Generation: 10 MHz (100 ns period)

//-------------------------------------------------------------------------

initial begin

CLK_TB = 0;

forever #(CLK_PERIOD / 2) CLK_TB = ~CLK_TB;

end

//-------------------------------------------------------------------------

// Timeout Watchdog

//-------------------------------------------------------------------------

initial begin

#100000;

$display("[TIMEOUT] Simulation exceeded limit. Terminating.");

$finish;

end

//-------------------------------------------------------------------------

// Tasks

//-------------------------------------------------------------------------

task do_reset;

begin

RST_TB    = 1'b0;

ACTIVE_TB = 1'b0;

DATA_TB   = 1'b0;

@(negedge CLK_TB);

@(negedge CLK_TB);

RST_TB = 1'b1;

@(negedge CLK_TB);

end

endtask

task drive_byte;

input [No_Registers-1:0] byte_in;

integer i;

begin

ACTIVE_TB = 1'b1;

for (i = 0; i < No_Registers; i = i + 1) begin

DATA_TB = byte_in[i];      // LSB first

@(negedge CLK_TB);

end

ACTIVE_TB = 1'b0;

end

endtask

task capture_and_check;

input [No_Registers-1:0] expected;

input integer idx;

reg   [No_Registers-1:0] received;

integer i;

begin

@(posedge Valid_TB);

for (i = 0; i < No_Registers; i = i + 1) begin

@(negedge CLK_TB);

received[i] = CRC_TB;      // R0 first -> LSB, R7 last -> MSB

end

if (received === expected) begin

$display("Test %0d PASSED  (expected=%h, got=%h)", idx, expected, received);

passed = passed + 1;

end else begin

$display("Test %0d FAILED  (expected=%h, got=%h)", idx, expected, received);

failed = failed + 1;

end

end

endtask

//-------------------------------------------------------------------------

// Main stimulus

//-------------------------------------------------------------------------

initial begin

$dumpfile("CRC_DUMP.vcd");

$dumpvars;

$readmemh("DATA_h.txt", DATA_input);

$readmemh("Expec_Out_h.txt", Expected_Out_CRC);

for (test_num = 0; test_num < No_Tests; test_num = test_num + 1) begin

do_reset;

drive_byte(DATA_input[test_num]);

capture_and_check(Expected_Out_CRC[test_num], test_num);

end

$display("---------------------------------------");

$display("TOTAL: %0d passed, %0d failed out of %0d", passed, failed, No_Tests);

$display("---------------------------------------");

#(CLK_PERIOD);

$finish;

end

endmodule //CRC_TB

Simulation results for all 10 test vectors:

| Test # | Input DATA byte (bin / hex) | Expected CRC (hex) | Simulated CRC (hex) | Result |
| --- | --- | --- | --- | --- |
| 0 | 10010011 (0x93) | 0x78 | 0x78 | PASS |
| 1 | 01110010 (0x72) | 0x44 | 0x44 | PASS |
| 2 | 00110110 (0x36) | 0x11 | 0x11 | PASS |
| 3 | 00011011 (0x1B) | 0xd2 | 0xd2 | PASS |
| 4 | 10100110 (0xA6) | 0x09 | 0x09 | PASS |
| 5 | 11000000 (0xC0) | 0xb2 | 0xb2 | PASS |
| 6 | 01010101 (0x55) | 0x36 | 0x36 | PASS |
| 7 | 11110010 (0xF2) | 0x80 | 0x80 | PASS |
| 8 | 01011110 (0x5E) | 0x2c | 0x2c | PASS |
| 9 | 00010001 (0x11) | 0x63 | 0x63 | PASS |

Console summary from the ModelSim transcript:

Waveform snapshots from ModelSim, showing DATA being shifted in and the CRC/Valid outputs being shifted out for the first test cases:

Figure 3: Waveform for Test 0 — DATA_input[0]=8'h93, LFSR reset to SEED (D8), CRC/Counter progressing through the shift-out phase.

Figure 4: Waveform for Test 1 — LFSR content after loading DATA_input[1]=8'h72, matching Expected_Out_CRC[1]=0x44.

Figure 5: Waveform for Test 2 — passed/failed counters incrementing (passed=2, failed=0) as successive tests complete.

# 05  DISCUSSION

All 10 test cases passed with the simulated CRC output matching the expected value exactly (10 passed, 0 failed), confirming that the LFSR correctly implements the specified feedback polynomial (taps after R6 and R2) and that the shift-in / shift-out ordering (DATA LSB-first in, CRC R0-first out) matches the specification.

One subtlety worth noting is the timing of Valid and Counter: because Counter is only re-evaluated on the clock edge after ACTIVE deasserts, the first CRC bit becomes valid one cycle after ACTIVE goes low, which the testbench correctly handles by waiting on the positive edge of Valid before sampling. No timing violations, race conditions, or mismatches were observed across the 10 vectors, and the asynchronous active-low reset correctly re-initialised the registers to the seed (8'hD8) before every test.

# 06  CONCLUSION

An 8-bit Galois LFSR-based CRC generator was designed in Verilog, matching the specified seed, feedback taps, and serial shift-in/shift-out protocol, and was verified against 10 self-checking test vectors in ModelSim with a 100% pass rate. The exercise reinforced how LFSR feedback taps determine the generator polynomial, how registered outputs affect output timing relative to a Valid flag, and how a file-driven, self-checking testbench simplifies regression-style verification of serial protocols.